Chapter 2 – Force Vectors

2.1 Scalars and Vectors

Vectors – A mathematical quantity possessing magnitude and direction.

Scalar – A mathematical quantity possessing magnitude only.

Name some vectors: forces, velocity, displacement

Name some scalars: Area, volume, mass energy

Representation of vector

Bold  R – Word Processors  Book uses this.

Arrow R– Long Hand, Word Processors

Underline  R – Long Hand, Typewriter, Word Processors

Magnitude of a Vector

R	Book uses Italics for all scalars

Types of Vectors

Fixed (or bound) vectors – a vector for which a unique point of application is specified and thus cannot be moved without modifying the conditions of the problem.

Free vector – a vector whose action is not confined to or associated with a unique line in space. (couple)

Sliding vector – a vector for which a unique line in space (line of action) must be maintained.

For 2 vectors to be equal they must have the same:

Magnitude	     P	 P

Direction

They do not need to have the same point of application.

A negative vector of a given vector has the same magnitude but opposite direction.

P	      -P

P and –P are equal and opposite P + (-P) = 0

2.2 Vector Operations

Product of a scalar and a vector

P + P + P = 4P  (the number 4 is a scalar)

This is a vector in the same direction as P but 4 times as long.

(+n)P = vector same direction as P, n times as long

(-n)P = vector opposite direction as P, n times as long

Vector addition

The sum of 2 vectors can be obtained by attaching the 2 vectors to the same point and constructing a parallelogram – Parallelogram law.

R

Q

R = P + Q

P		R = resultant vector

Note: The magnitude of P + Q is not usually equal to P + Q.

Addition of vectors is communative: P + Q = Q + P

Triangle Rule

P

Q	   R	 R	Q

P

Let’s add 3 vectors!

R1

R1Parallelogram Law

R		       Q			            Q

Q

P		            P			        P

S		           S			    S

R1 = Q + P			R = R1 + S = Q + P + S

Triangle Rule

Q		       Q	   P			P                 S

R1               Q	       R1

P					R

S		     S			R = R1 + S = Q + P + S

R1 = Q + P

Polygon Rule – Successive applications of triangle rule.

Q

Q                        P

S

P                                   R

S

Note: P  +Q + S = (P + Q) + S = P + (Q + S) vector addition is associative

Vector Subtraction – the addition of the corresponding negative vector

P – Q = P + (-Q)

Resolution of vector into components

A single vector can be represented by 2 or more vectors.  These vectors are components of the original vector.  Finding these is called resolving the vector into its components.

There is an infinite number of ways to resolve one vector.

etc.

2 cases of particular interest are:

1). One of the 2 components is known.   Easy (see above)

2). The line of action of both components is know.

Let’s look at #2 more closely.  When would this happen?

When you are given a coordinate system!

What are the x and y components of P IF

P = 1000 lbs  = 30o		Px = P cos 30o = 866 lbs

## Py = P sin 30o = 500 lbs

Note: Given Px and Py, what is P?

P2 = Px2 + Py2=8662 + 5002 = 1000 lbs

Homework: Examples 2-1 through 2-4 are really good, Understand them!

Problems: 2-1, 2-4, 2-6, 2-13, 2-21, 2-25, 2-30

Read: Section 2.4

1).	Given: The fixed structure shown below.

P = 500 N

T = 200 N

Find: Combine P and T into a single force R

tanα=BDAD=5sin75∘3+5cos75∘α=48.4∘

Law of cosines:

c2=a2+b2−2abcos(c)R2=2002+5002−2(200)(500)cos(48.4∘)R=396.5 N

Law of sines:

200sinθ=396.5sin48.4∘		θ=22.2∘

R=396.5 N	              22.2∘

- Given: A barge is pulled by 2 tugboats.  The resultant of the forces exerted by the

tugboats is a 5000 pound force directed along the center axis of the barge.

A

1

30∘

B



C

2

Find:	a). tension in each rope if  =45 degrees

b). value of  such that the tension in rope 2 is minimum.

5000

a).

45∘	 30∘

T2	     T1

T1sin45∘=T2sin30∘=5000sin105∘

T1=3660 lbsT2=2590 lbs

b).		5000

dir of T1

30o

Dir of T2?

Dir of T2?

Dir of T2?

α=60∘T1=5000cos30∘=4330 lbsT2=5000sin30∘=2500 lbs

3).	Given: Problem 2-10 Pg., 27

45∘

60∘350 lbs

FABsin60∘=FACsin45∘=350sin75∘

FAB=314 lbsFAC=256 lbs