# Chapter 4-Force System Resultants

In Chap. 2 we showed that ΣF=0 was the only condition for the equilibrium of a particle.  Now, what about a rigid body?

F

F

ΣF=0, but not in equilibrium.  The body rotates. So ΣF=0is a necessary condition, but not sufficient.  There is a Torque or Moment on the body.  This is what will be discussed in this chapter.

4.1 Cross Product (Vector Product)

The cross product of 2 vectors P and Q is defined as the vector Vwhich satisfies:

1). The line of action of Vis perpendicular to the plane containingP and Q.

2).  The magnitude of Vis given by:

V = Pqsin  (≤0≤180∘)

3).The direction of V follows the right-hand rule.

V=P×Q

Q



P

V=P×Qread as "V equals P cross Q"

Laws of Operation

Communicative law

P×Q≠Q×P(note:P×Q=−Q×P)

Multiplication by a Scalar

a(A×B=(aA×B+A×(aB)=(A×B)a

Distributive law

P×(Q1+Q2)=P×Q1+P×Q2

Associative Law

(P×Q)×S≠P×(Q×S)

Cartesian Vector Formulation

What is i×j?

j

i																																																          k

Magnitude: i×j= (1)(1)sin90∘= 1

Direction: +k

i×i=0i×j=ki×k=j	j×i=kj×j=0j×k=i	k×i=jk×j=ik×k=0

Here is a simple way of remembering this.

j

k

i

+

Now we will express V of V=P×Q in the terms of rectangular components

V=P×Q=(Pxi+Pyj+Pzk)×(Qxi+Qyj+Qzk)

Using the distributive and scalar mult. properties.

V=PxQx(i×i)+PxQy(i×k)+PyQx(j×i)+PyQy(j×j)+PyQz(j×k)+PzQx(k×i)+PzQy(k×j)+PzQz(k×k)

V=0+PxQy(k)+PxQz(−j)+PyQx(−k)+0+PyQz(i)+PzQz(i)+PzQy(−i)+0

V=(PyQz−PzQy)i+(PzQx−PxQz)j+(PyQy−PyQx)k

Vx		Vy		Vz

The vector product can be more easily memorized.

V=ijkPxPyPzQxQyQz

Does everyone remember how to do determinants?

ithTerm PYQZ−QyPZ=PYQZ−PZQYjthTerm −(PXQZ−QXPZ)=PZQ−PXQkthTerm PXQY−QXPY=PXQY−PYQalignlX		Same as VX VY VZ

4.2 Moment of a force-scalar formulation

Def.: Moment- The measure of the tendency of the force to make a rigid body rotate about a point or fixed axis.

F

M∘																												     	 																								  													  R	A											         O	      													     d		A- Point of application ofF

Def: The Moment of F about O is defined as:

M0=R×F

The direction of Mois also defined as the direction which would bring R in line with F right hand rule.

Note: Ris a vector from O to any point on the line of action of F.

From the definition of a cross product

M0=rFsinθM0=Frsinθ

From our diagram , r sin=d, so

M0=Fd			Units: Nm, ft lbs, in lbs

Where d represents the perpendicular from O to the line of action of F.  d is commonly known as the moment arm.

4.3 Moment of a force- vector formulation

Remember:   M0=R×F

We also know:  R=Rxi+Ryj+RzkF=Fxi+Fyj+Fzk

Thus,

M0=ijkrxryrzFxFyFz

Again, use vectors in 3D!!

Resultant moment of a system of forces

MR0=Σ(R×F)

1). Given

A

24"		100lb

60o

0

Find: 	a). Moment of the 100 lb force about 0.

b). Magnitude of a horizontal force applied at a which create the same moment about 0.

c). The smallest force applied at a which creates the same moment about 0.

d). Distance from 0 a 250lb vertical force must act to create the same moment about 0.

A

a)M0=Fd=100(24cos60∘)M0=1200 inlbs.	 24"	     100

60∘

M∘       0	------------

P

60	     P

b).					     30

L		24

M∘

60

0

M0=Fd

2 Ways

- Components

M0Pperpd=>1200=Pcos30∘(24)

P=57.7 lbs

- Perpendicular line to force

M0=Pl=>1200=P(24cos30∘)

P=57.7lbs

A

c).			  P

M∘      60∘

M0=Fd1200=P(24)P=50 lbs             0

Why is P perpendicular to the lever?

-P is smallest when d in M=Fd is a maximum. This occurs

when P is perpendicular to the lever.

-If P is not perpendicular to the lever, we have components parallel and perpendicular to the lever, thus the parallel component is "robbing" the perpendicular component of some force which could contribute to creating the moment.

A

M0=Fd1200=240(dcos60∘)d=10ind).

B

240

M∘	    60∘

0

2).  Given: The rectangular platform is hinged at A and B and supported by a cable which passes over A frictionless hook at E.  The tension in the cable is 1349N.

Find: The moment about each of the coordinate axes of the force exerted by the cable at C.

rOC=0i+0j+2.25 {k

3-D use: MC=rOC×FC

FC=426 {i+710 {j−1065 {kFC=FCuCE=1349(0.9−0)i+(1.5−0)j+(0−2.25)k0.92+1.52+2.252

MC=ijk002.25426710−1065=0+0+2.25(−1)(2+3)[710 {i−426 {j]

MC=−1598 {i+959 {j+0k Nm

Mx=1598 NmMy=959 NmMz=0 Nm

Note:  MC=rOE×FC could have been used also.

MC=ijk0.91.50426710−1065=0.9(−1)2+1[−1065 {j−710 {k]+1.5(−1)2+2[−1065 {i−426 {k]

MC=959 {j+639 {k+1598 {i−639 {kMC=1598 {i+959 {j+0k