4.4 Principle of Moments

Also known as Varigon's Theorem

Let's say we have several concurrent forces. We can determine its resultant

R=F1+F2...

The moment of Rabout 0, where ris the position vector from 0 to A point on the line of action of R.

MO=r×R]MO=r×(F1+F2+...)MO=r×F1+r×F2+...

The moment about a given 0 of the resultant of several concurrent forces is equal to the sum of the moments of the various forces about the same point 0.

4.5 Moment of a force about a specified axis

λ										           y

F

MO			          C

u

r			   	      A														        x										        0																																						          z

Def: The moment M of Fabout λis the projection OC of the moment MO on the axis λ.

Mλ=u⋅MO=u⋅(r×F)   A triple scalar product

May be written:

Mλ=u⋅(r×F)=uxuyuzrxryrzFxFyFz

4.6 Moment of a couple

Def: Couple - 2 forces F and −F having the same magnitude, parallel lines of action, and opposite direction.

d	d

F−F																		ΣFy=0    but will rotate!

Suppose we have

M=rA×F+rB×(−F)

Summing moments about O.

M=(rA−rB)×F

rB		               B

rF		   				     But:  rB+r=rA ⇒ r=rA−rB

- −F			     Thus:   M=r×F

rA

A

Note that the right hand side has NO reference to 0, thus Mis the same for ANY moments center.  M is a free vector.

Think of it this way: F  creates a moment about some point, −F, creates another moment about the same point, usually in the opposite direction. The difference between these 2 moments is

always the same.

As long as the magnitude and direction of M does not change, you have equivalent couples.

In the scalar world:

M=Fd		    d

F

Homework:	Read 4.7 – 4.9

Problems: 4-7, 25, 51, 70, 76, 85, 87

1). Given: A piece of plywood in which several holes are being drilled has been secured to a workbench by means of 2 nails. The drill exerts a 12 Nm torque on the piece of plywood.

A															C																									B											450mm		240mm

Find: The resulting forces applied to the nails if they are located at:

a). A and B

b). B and C

c). A and C

a)

M=Fd12=F(0.45)F=26.7N														      12Nm													         F																						              	F       450mm

b).

M=Fd12=F(0.24)F=50N										                      	240mm

12Nm

M=Fd12=F(0.51)F=23.5Nd=0.452+0.242=0.51c).

2). Given: The rod shown below is supported by two brackets at A and B. Given that the magnitude of Fis 700 N.

A (0,0,0,)

B (0.4,0.2,0)

C (0,6,0,0.3)

D (0,0.2,0)

Find: The moment about axis AB MABdue to F.

MAB=uAB.(rAC×F) rAC=0.6i+0j+0.3k

F=FuCD=700(0−0.6)i+(0.2−0)j+(0−0.3)k0.62+0.22+0.32

F=−600 {i+200 {j−600 {k

uAB=0.4i+0.2j+0k0.42+0.22=0.894 {i+0.447 {j

MAB=0.8940.44700.600.3−600200−300=0.894[(0)(−300)−(200)(0.3)]−0.447[(0.6)(−300)−(−600)(0.3)]+0

MAB=−53.64 Nm

3). Given: F=i+3j lbs

y		B

F

0	  x

−F							A (4,-1)

B (3,2)

A			0' (2,-2)

0'

Find: MO and MO'due to the couple

MO=rOA×F+rOB×(−F))		rOA=4i−j		rOB=3i+2j

rOA×F=ijk4−10130=0i+0j+(4(3)−(−1)(1))k=13 {k   ft lbs

rOB×(−F)=ijk320−1−30=0i+0j+(3(−3)−(−1)(2))k=−7k   ft lbs

MO=13 {k−7k ⇒ MO=6k ft lbs

MO=rBA×F		rBA=i−3j

rBA×F=ijk1−30130=0i+0j+(1(3)−(1)(−3))k=6k   ft lbs

MO'=rO'A×F+rO'B×(−F))		rO'A=2i+j		rO'B=i+4j

rO'A×F=ijk210130=0i+0j+(2(3)−(1)(1))k=5k   ft lbs

rO'B×(−F)=ijk140−1−30=0i+0j+(1(−3)−(−1)(4))k=1k   ft lbs

MO'=5k+1k ⇒ MO'=6k ft lbs