4.7-4.9 Force Couple Systems

Let us look at the following

F

r  			           A																								    0

We want for one reason or another, to have F act at point 0.  How do we do this?

We can move F along its line of action, but we cannot just move it to 0 because if we do so we will modify the action of F on the body.

Let do the following

F

F

d

r  			           A																								    0

−F

We haven't changed anything

FMO

MO=r×F		Moment of a couple!

M=Fd       Magnitude

0

Thus any force Facting on a rigid body may be moved to an arbitrary point 0, provided that a couple of moment equal to the moment of Fabout 0 is added.

Since M is a free vector it may be applied anywhere.  For convenience the couple vector is

usually attached at 0.

This is known as a force-couple system.

The opposite can also be done. Any force-couple system  may be replaced by a single equivalent force.  How is this done?

This is done by moving the force F in the plane perpendicular to M until its moment about 0 becomes equal to the moment of the couple to be eliminated.

0B1). Given

60∘										300mm																						200 N					  													    400 N										000												O

60 mm				    200 N

Find:  replace the couple and force by an equivalent, single force applied to the lever.  Determine the distance from 0 to the point of application of this force.

B

400N

60Nm	=     84	Nm	=	   000		                    24Nm			=	  24Nm																										400N			  400N

B

400N			M=Fd

84 = 400(OC cos 60∘)

=					OC = 0.42 m

60o					OC = 420 mm

d