# Chapter 5- Equilibrium of a rigid body

5.1 Conditions for rigid-body equilibrium

Previously we found that a system of forces can be reduced to a force-couple system.

When the force and couple are equal to zero, the body is said to be in equilibrium.

The necessary and sufficient conditions for the equilibrium of rigid body are:

ΣF=0	ΣM=0

In component form:

ΣFX=0ΣFY=0ΣFZ=0	ΣMX=0ΣMY=0ΣMZ=0

Equilibrium in two dimensions

5.2 Free-body diagrams

Drawing a FBD, steps

1).  Decide which body to analyze.

External  	2).  Separate this body from everything else and sketch the contour,

forces		3).  Draw all applied forces (weight).

4).  Draw all reactions.

5). Include any necessary dimensions and coordinate axis.

If you don't know a direction assume a direction and let the sign of the answer tell you if the direction is correct or not.

Rules:

- The magnitude and direction of known forces should be clearly indicated (usually applied forces)

- Indicate the direction of the force exerted on the body, not the force exerted by the body.

Unknown forces are usually the reactions (constraining forces).

Reactions are supports and connections in 2-D, pg. 184-185.

The best 2 ways I have found to determine reactions are:

- Remove the support and see what happens

FBD		 y	 x																				      N																						mg

2. Look at a support and "pull" on it.

0											                    	                                       				                 y

0				    Rx					            x

Ry

If I pull on the rod, it can't move in x, y direction, but I can make it rotate about 0.

  - Equations of equilibrium

y

ΣFX=0ΣFY=0ΣFZ=0	ΣMX=0ΣMY=0ΣMZ=0

x																										 3 equations, 3 unknowns

Let's look at a truss

D			   Q		   S

C				      D

A	   			    	  0     B

P	         Q		  S

FBD

W											AX

AY		       BY

ΣMA=0=>BYΣFX=0=>AXΣFY=0=>AY

Additional equations could be written.

ΣMB=0 Does not provide any new info.

This is not an independent equation.

You can use ΣMB=0to replace one of the above 3.

ΣFX=0ΣMA=0ΣMB=0	or	ΣMA=0ΣMB=0ΣMC=0

Homework:

Read:

Problems 5-12, 19, 23, 27, 30, 38

1). Given

Find: Reactions

ΣFx=02+2+RAx=0RAx=−4 kNΣFy=0−3+RAy+RB=0FBD

ΣMA=0−2(3)−2(1.5)+RB(2)=0RB=4.5 kN						Substituting into RB y-equation:    RAy = -1.5   kN

2). Given

Find: Reactions at A

ΣFx=0RAx=0ΣFy=0RAy−2000=0RAy=2000 lbsFBD

ΣMA=0MA+3000−2000(5)=0MA=7,000 ft lbs

3). Given:  The bar AB weighs 250 pounds and all surfaces are smooth.

Find: Cable tension and forces at A and C.

ΣFx=0F−RCcos40=0F−0.766RC=0ΣFy=0RA−W+RCsin40=0RA+RCsin40=250RA+0.643RC=250FBD

ΣMA=0−F(2sin50)−W(6cos50)+RC(8)=0−1.532F+8RC=964.2

Solving the above 3 equations simultaneously:

RA = 159.2    lbs

F = 108.2    lbs

RC = 141.2    lbs

4). Given: The loading car weight is 5500 lbs. and its CG is at point G.

Find: tension in cable and reactions at wheels.

FBD

To find R2, sum moments about A to eliminate T and R.

ΣMA=0R2(50)−Wx(6)−Wx(25)=050R2−5500cos25∘(6)−5500sin25∘(25)=0R2=1760 lbs

To find R1, sum moments about B to eliminate T and R2

ΣMB=0−R1(50)−5500cos25∘(6)+5500sin25∘(25)=0R1=564 lbs

To find T:

ΣFx=0−T+WX=0T=5500cos25∘T=4985 lbs

Check:

ΣFx=0564+1760−5500sin25∘=0−0.4≈0

5).  Given: The tension in DF is 150 kN

Find: Determine the reactions at E (Pt. E is fixed).

θΣFx=0−Ex+150sinθ=0Ex=1504.57.5Ex=90 kNDF=4.52+62FBD

ΣME=0ME+20(1.8)(4)+20(1.8)(3)+20(1.8)(2)+20(1.8)−150sinθ(6)=0ME=1504.57.5(6)−360ME=180 kN mΣFy=0−80+Ey−150cosθ=0Ey=80+15067.5Ey=200 kN

6). Given: the 10 ton moving crane shown below has a mass center at C. It carries a maximum load of 18 tons.

Find:  a). The smallest weight of counterweight C, and also the largest distance D, so that:

- the crane doesn't tip cw when the maximum load is lifted

- the crane doesn't tip ccw when there is no load

b). The range of weights C for which (i) and (ii) can be satisfied if d = 2.5 feet.

ΣMB=0WC(d+2)−10(1)−18(13)=0WC(d+2)=244WCd+2WC=244ΣMA=0WC(d−2)−10(5)−0(17)=0WC(d−2)=50WCd−2WC=50a).

Simultaneously solving the above two equations yields:

Wc = 48.5    tons

d = 3.03    feet

ΣMB=0WC(2.5+2)=244WC=54.2 tonsΣMA=0WC(2.5−2)=50WC=100 tonsb).

Thus, the range of weights is:

- 54.2≤WC≥100 tons

7)  Given: the spring is unstretched when = 0, and the spring constant k = 250 lb/in.

Find: Determine the position of equilibrium

FBD

Force of a spring: F = ks	s = deflection of spring

Deflection of spring:	s = r		Arclength

F = kr

ΣMO=0W(8sinθ)−rF=0400(8sinθ)−r(krθ)=03200sinθ−(3)(250)(3)θ=03200sinθ−2250θ=0

How do we solve this

-Graphing	y = 3200 sin- 2250

-Iteratively