Composite plates and wires

In most cases a flat plate can be divided up into common shapes.  We can use this fact to find the center of gravity and/or the centroid.

x

y

Equating moments

ΣMy:X ΣW=X(W1+W2+......Wn)=x1W1+x2W2+....+xnWnΣMx:Y ΣW=Y(W1+W2.......+Wn)=y1W1+y2W2+.....ynWn

X ΣW=Σ xWY ΣW=Σ yWZ ΣW=Σ zW		Solve for C.G.

Following the previous development

X ΣΑ=Σ xAY ΣΑ=Σ yΑZ ΣΑ=Σ zA	Solve for centroid

Examples

1). Calculate the x and y coordinate of the centroid of the shaded area.

| Part # | Area (mm2) | x (mm) | y (mm) | xA (mm3) | yA (mm3) |
| --- | --- | --- | --- | --- | --- |
| 1 | /4)(240)2 = 45,239 | 0 | 0 | 0 | 0 |
| 2 | -0.5(180)(90) = -8,100 | -90/3 = -30 | 0 | 243,000 | 0 |
|  | ΣA=37,139 |  |  | Σ xA=243,000 | Σ yA=0 |

Centroid:

X ΣA=Σ xAX(37,139)=243,000X=6.5 mmY ΣA=Σ yAY(37,139)=0Y=0 mm

2).  Find the y coordinate, measured up from the bottom, of the area shown below.

| Part # | Area (mm2) | y (mm) | yA (mm3) |
| --- | --- | --- | --- |
| 1 | 10(160) = 1,600 | 5 | 5(1600) = 8,000 |
| 2 | 2[10(30)] = 600 | 15 | 15(600) = 9,000 |
| 3 | 2[10(50)] = 1,000 | 35 | 35(100) = 35,000 |
| 4 | 2[10(120)] = 2,400 | 70 | 70(2,400) = 168,000 |
|  | ΣA=5,600 |  | Σ yA=220,000 |

Centroid:

Y=Σ yAΣ A=220,0005600Y=39.3 mm

3) Given: the machine element shown below

= 25 kg/m3 =2.5(10−8) kg/mm3 = 2.5(10−5) g/mm3

Find: C.G.

MI=12(π8024)(12)(2.5)(10−5)=0.754 g		MIV=100(80)(12)(2.5)(10−5)=2.4 g

MII=π4824(12(2.5)(10−3)=0.543 g		MV=π4824(12)(2.5)(10−5)=0.543 g

MIII=80(50)(12)(2.5)(10−5)=1.2 g	     semicircle	y=4r3π=4(40)3π=16.98

| Part # | mass (g) | x (mm) | y (mm) | z (mm) | mx (mm3) | my (mm3) | mz (mm3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I | 0.754 | 6 | 78.98 | -40 | 4.524 | 59.55 | -30.16 |
| II | -0.543 | 6 | 62 | -40 | -3.258 | -33.666 | 21.72 |
| III | 1.2 | 6 | 37 | -40 | 7.2 | 44.4 | -48 |
| IV | 2.4 | 50 | 6 | -40 | 120.0 | 44.4 | -96 |
| V | -0.543 | 55 | 6 | -40 | -29.865 | -3.258 | 21.72 |
| ### Totals | 3.268 |  |  |  | 98.601 | 81.426 | -130.72 |

X=ΣxmΣm=98.6013.268			Y=81.4263.268		Z=−130.723.268

X=30.2 mm				Y=24.9 mm		Z=−40 mm