## Moments of inertia by composite areas

A composite area, A, made several components areas A1,A2,....

The integral representing the MOI of  A may be subdivided into integrals computed over A1,A2,...

Thus, the MOI of A with respect to a given axis can be obtained by summing the MOI of A1,A2,... with respect to the same axis.

MOI of common shapes Pg. 509-514

2 things:

1).  Before adding the MOI of the component areas, the parallel axis theorem should be used to transfer each MOI to the desired axis.

2).  The radius of gyration of a composite area is NOT equal to the sum of the

radii of gyration of the component areas.

Example

1).  Determine Ix and Iy of the area shown with respect to centroidal axis.

| Part # | A | y' | Ay' | Ix (mm4) | Adx2 (mm4) | Iy (mm4) | Ady2 (mm4) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4,800 | 40 | 192,000 | (60)(803)/12 = 2.56(106) | 4800(76-40)2 = 6.22(106) | (603)(80)/12 = 1.44(106) | 0 |
| 2 | 7,200 | 100 | 720,000 | 180(403)/12 = 960(103) | 7200(100-76)2 = 4.15(106) | (1803)(40)/12 = 19.44(106) | 0 |
|  | 12,000 |  | 412,000 | 3.52(106) | 10.37(106) | 20.88(106) | 0 |

Entire section:

Ix=ΣIx+ΣAd2=13.89(106) mm4

Ix=ΣIx+ΣAd2=20.88(106) mm4