Belt Friction

Let's look at a flat belt passing over a drum

Let's take a look at a differential element

Motion is assumed to be impending.

∴dF=μsdN

The normal force is a differential force because it acts on a differential element of area.

ΣFx=0−Tcosdθ2−μsdN+(T+dT)cosdθ2=0

ΣFy=0−Tsindθ2−(T+dT)sindθ2+dN=0

For small angles

cos(d) = 1

sin(d) = d

ΣFx ⇒ −T−μsdN+T+dT=0

dT=μsdN

ΣFy ⇒ −Tdθ2−(T+dT)dθ2+dN=0

−Tdθ2−Tdθ2−dTdθ2+dN=0

dTdθ2=0  (2 small numbers squared = 0)

Thus: dN=Tdθ

Substituting:

dT=μsTdθdTT=μsdθ

Integrating:

dTT=μsdθ

lnT2−lnT1=μSB

Thus:

lnT2T1=μsB

OR

T2T1=eμsB

These formulas apply to:

1).  Flat belts passing over fixed drums.

2).  Ropes wrapped around a post or capstan.

3).  band brakes- the drum is about to rotate while the band remains fixed.

4). Problems involving belt drives. Both the pulley and belt rotate; determine whether the belt will slip/move to the pulley.

-	Should only be used if the belt, rope, or brake is about to slip.

-	In the equations T2 will always be larger than T1.  So T2 represents the tension which pulls and   T1 is the resisting tension.

-	 must be in radius. B may be larger than 2π.  If a rope is wrapped around a post n times, β=2πn.

-	If the belt, rope is actually slipping, μk should be used.

-If no slipping occurs and is not about to slip, none of these formulas should be used.

V-belts

Proceeding as before:    T2T1=eμsβsin (α2)

Examples

1). Given: A hawser thrown from a ship to a pier is wrapped 2 full turns around a capstan.  The tension in the hawser is 7500 N; by exerting a force of 150 N on its free end, a dock worker can just keep the hawser from slipping.

Find: 	a).  Determine the coefficient of friction between the hawser and capstan.

b).  Determine the tension in the hawser that could be resisted by 150 N force if the

hawser were wrapped 3 full turns around the capstan.

- Slipping is impending

lnT2T1=μSBln7500150=μS2 turns2π radturnμS=0.311

b)

lnT2T1=μSBT2150=e(.311)(3)(2π)T2=52.7 kN

2).	Given: A flat belt is used to transmit the 30 ft lb torque developed by an electric motor. The

drum in contact with the belt has a diameter of 8 in and μS=0.30.

T2=TAT1=TBβ=160∘Find:  Determine the minimum allowable value of the tension in each part of the belt if the belt is not to slip.

T2T1=eμsβTΑTΒ=e0.38π9TΑ=2.311 TΒ

M=(TΑ−TΒ)r30=(TΑ−TΒ)8 in21 ft12 inTA−TB=90TΑ=TΒ+902.311TB=TB+90TB=68.6 lbsTΑ=158.6 lbs

3).	Given:  Example problem #2, using a V-belt with α=36∘

Find:  Determine minimum allowable tension

T2T1=eμsβ/sin(α2)TΑTΒ=e0.3(8π9)/sin18∘TΑ=15.045 TΒ			M=(TΑ−TΒ)r30=(TΑ−TΒ)412TA−TB=90TΑ=TΒ+90